# MATERIAL SAFETY DATA SHEET-DATED 10/1/09

- CHEMICAL PRODUCT & COMPANY INDENTIFATION

## TRADE NAME:      DS88K

MANUFACTURE:     DIP SEAL PLASTICS, INC.

2311 23RD AVE., ROCKFORD, IL 61104

## TELEPHONE NUMBER:800-634-7821

- HAZARDS INDENTIFICATION

POTENTIAL HEALTH EFFECTS, Skin-exposure to heated material can cause thermal burns.

Potential Health Effects, Inhalation-Overheated material may produce vapors which may cause mild respiratory irritation.

Potential Health Effects,Eye-MELTED MATERIAL MAY CAUSE SEVERE BURNS.

- FIRST AID MEASURES

Skin-IF MOLTEN PLASTIC CONTACTS WITH SKIN, REMOVE IMMEDIATLEY AND APPLY COLD WATER, ICE OR COLD COMPRESS.

Eye-GET IMMEDIATE MEDICAL ATTENTION.

Ingestion-SMALL AMOUNTS ARE ESSENTIALLY HARMLESS.

- FIRE FIGHTING MEASURE

Extinguishing Media-Foam, CO2, Dry chemical

Basic Fire Fighter Procedures-Fire fighters must wear                                        MSHA/NIOSH approved positive pressure breathing apparatus (SCBA) with full facemask and full protective equipment.  Do not use water on molten product to avoid splattering and spread of fire.

Flash Point: 400 F

- ACCIDENTAL RELASE MEASURES

Spill or Leak Procedure-Allow material to solidify and scrape up.

- HANDLING & STORAGE

Storage-Store in a cool, dry place.

- EXPOSURE CONTROLS/PERSONAL PROTECTION

Skin Protection: Personal Protection Equipment (PPE)-When material is heated; wear gloves to protect against thermal burns.

Eye Protection: Personal Protection Equipment (PPE)-Wear chemical safety goggles and face shield.

Respiratory Protection: Personal Protection Equipment (PPE)- None normally needed.

8.   PHYSICAL & CHEMICAL PROPERTIES

Specific Gravity: .91+/-.03  Melting Point: 193 degrees F  Solubility in Water: Not soluble

Chemical Family:  Butyle acetate

9.    STABILITY & REACTIVITY

Stability/Incompatibility-Stabl under normal conditions of use.

Hazardous Reactions/Decomposition Products-If product is burned, carbon monoxide, and other unknown product may be produced.  Avoid the addition of water or any volatile material to molten product.

10.  DISPOSAL CONSIDERATIONS

Waste Disposal-Not listed as a material banned from land disposal according to RCRA.  Observe all federal, state, and local environmental regulations.

11.  REGULATORY INFORMATION

HMIS RATINGS: Health-1 Flammability-1 Reactivity-0   Personal Prot. Equip.-B